IronSignature

hypothesis:

Iron deficiency alters peripheral tissue properties (perfusion, impedance, autonomic tone) in ways that measurably affect dynamic electrical coupling during controlled capacitive interactions.

Data collection:

A custom Android research application executed privileged commands on a rooted device to access raw touchscreen mutual-capacitance data exposed by the kernel driver. This design allowed precise data capture while enabling controlled user interaction tasks

How to reduce confounds:

  Fix finger location with an on-screen ring + crosshair

  Use same finger (e.g., right index) always

  Don’t allow USB/charging

## What iron deficiency could realistically change

Iron deficiency → anemia → reduced hemoglobin → altered peripheral circulation.

That translates (physically) into four possible effects on capacitive touch.

I’ll map each one to something you can actually measure.

## 1️⃣ Slower stabilization after contact

### What you’d see

- Instead of stabilizing quickly, the curve settles more slowly

- First 3–6 points drift before flattening

### Why

- Reduced blood oxygenation affects microvascular response

- Tissue impedance equilibrates more slowly

### Feature

- Stabilization time
(how many samples until slope ≈ 0)

## 2️⃣ Different steady-state level (small, not dramatic)

### What you’d see

- The flat region sits slightly higher or lower

- Difference is subtle (few units, not orders of magnitude)

### Why

- Blood volume fraction affects effective dielectric constant

- Iron deficiency often correlates with altered perfusion

### Feature

- Mean delta after stabilization

⚠️ Alone, this is weak evidence — must combine with others.

## 3️⃣ Increased jitter / variability (very important)

This is the most plausible effect.

### What you’d see

- The points bounce more from sample to sample

- Same mean, but larger scatter

### Why

- Iron deficiency is associated with:

  - autonomic instability

  - reduced vascular responsiveness

  - subtle tremor/fatigue effects

### Feature

- Standard deviation over time

- Short-term jitter (std over sliding window)

This often separates groups better than the mean.

## 4️⃣ Slower recovery after pressure

If you add a press–release phase:

### What you’d see

- After pressing harder, the signal returns to baseline more slowly

- The recovery curve is shallower

### Why

- Capillary refill and tissue elasticity differ

- Anemia affects oxygen delivery and recovery dynamics

### Feature

- Recovery time constant

- Area under the recovery curve

This is a strong, physiologically grounded feature.